THE UNIVERSITY OF AZAD JAMMU AND KASHMIR, MUZAFFARABAD

| STUDENT NAME: | SHAHZAD AHMED AWAN |
| --- | --- |
| STUDENT ROLL NO: | 2024-SE-15 |
| LAB: | 06 |
| COURSE TITLE: | Computer Networks |
| COURSE CODE: | CS-2203 |
| INSTRUCTOR: | Engr. Dr. Asma Javaid |
| SUBMISSION DATE: | 07 April, 2026 |

Department of Software Engineering

# Lab 6: switch communication between two LAN

# 1. Objective

To interconnect two Layer-2 switches in a LAN, connect six PCs across them, and verify end-to-end communication using ICMP in Cisco Packet Tracer Simulation Mode.

# 2. Apparatus

| Item | Specification | Qty |
| --- | --- | --- |
| Switch | Cisco 2960 | 2 |
| PC | Generic (Packet Tracer) | 6 |
| Cable | Copper Straight-Through | 7 |
| Software | Cisco Packet Tracer | 1 |

# 3. Theory

A switch forwards frames within a LAN using MAC address tables. Two interconnected switches form a single broadcast domain — PCs on different switches communicate freely within the same IP subnet. ICMP ping confirms reachability. STP (802.1D) runs automatically to prevent loops; STP events in the simulation are normal behavior.

# 4. Network Topology

| Connection | Port |
| --- | --- |
| PC0 → Switch0 | Fa0/1 |
| PC1 → Switch0 | Fa0/2 |
| PC2 → Switch0 | Fa0/3 |
| PC3 → Switch1 | Fa0/1 |
| PC4 → Switch1 | Fa0/2 |
| PC5 → Switch1 | Fa0/3 |
| Switch0 → Switch1 (uplink) | Fa0/24 ↔ Fa0/24 |

# 5. IP Address Configuration

| Device | IP Address | Subnet Mask |
| --- | --- | --- |
| PC0 | 192.168.1.1 | 255.255.255.0 |
| PC1 | 192.168.1.2 | 255.255.255.0 |
| PC2 | 192.168.1.3 | 255.255.255.0 |
| PC3 | 192.168.1.4 | 255.255.255.0 |
| PC4 | 192.168.1.5 | 255.255.255.0 |
| PC5 | 192.168.1.6 | 255.255.255.0 |

All devices share the 192.168.1.0/24 subnet. No default gateway required.

# 6. Procedure

- Open Cisco Packet Tracer. Place 2 × Cisco 2960 switches and 6 × PCs on the workspace.

- Connect each PC to its switch using Copper Straight-Through cables (ports as listed above). Connect Switch0 Fa0/24 → Switch1 Fa0/24 for the uplink.

Figure 1: End PCs IP Address Configuration

For each PC: click PC → Desktop → IP Configuration → Static. Enter the IP address and subnet mask from the table above.

- Wait for all link LEDs to turn green (STP convergence). Switch to Simulation Mode (bottom-right). Ensure ICMP is enabled in Event List filters.

- Select Add Simple PDU (envelope icon). Click PC5 as source, PC1 as destination. Press Play to run the simulation.

Figure 2: Stimulation on the Network Designed

# 7. Results

Packet flow observed in the Simulation Event List:

| Time (s) | Last Device | At Device | Type |
| --- | --- | --- | --- |
| 0.000 | PC5 | Switch1 | ICMP |
| 0.001 | Switch1 | Switch0 | ICMP |
| 0.002 | Switch0 | PC1 | ICMP |
| 0.003 | PC1 | Switch0 | ICMP |
| 0.004 | Switch0 | Switch1 | ICMP |
| 0.005 | Switch1 | PC5 | ICMP |

Figure 3:  Simulation Event List showing all ICMP entries above.

PC5 → Switch1 → Switch0 → PC1 → Switch0 → Switch1 → PC5

# 8. Conclusion

Two Cisco 2960 switches were successfully interconnected and six PCs were configured within the 192.168.1.0/24 subnet. ICMP testing in Simulation Mode confirmed full bidirectional connectivity across both switches, demonstrating that Layer-2 switching enables seamless LAN communication without requiring a router.